Write a 200–250 word reflection discussing design choices, including
decoupling capacitor placement, crystal positioning, and ground plane
implementation

Decoupling capacitor placement as close as possible to the MCU’s power pins, with minimal trace length and low inductance paths to ground. 
Poor placement increases supply noise and voltage ripple, which can lead to erratic behavior, timing errors, or even unexpected resets under dynamic load conditions.

The crystal and its load capacitors should be placed very close to the MCU, with short, symmetric traces and minimal exposure to noisy signals. Routing these traces over a continuous ground plane helps reduce electromagnetic interference.

Using an uninterrupted ground plane on one layer provides a low-impedance return path and shields sensitive signals. Fragmented or poorly stitched ground planes can create ground loops, increase EMI susceptibility, and degrade analog and digital performance.